Re-Code for Enhanced Bluetooth:

Change FA (VO) to Remove 6NH from VO & Add 6NS to VO

VO Code HU_NBT

Read FA(VO)Activate FA(VO)Read SVTRight-Click ECU & Select Code

Now Proceed to code HU_NBT Module … or re-code if coding was already present

***If no Navigaion Option … replace HU_NBT with CMB_MEDIA & HU_ENTRY

ACSM:

Disable Welcome 3 Gongs:

3000  Initialwarnung_GWF_IW = nicht_aktiv

Turn Off Seat-Belt Reminder Chimes:

3000  SeatBeltReminder_SBR_Fahrer = nicht_aktiv

3000  SeatBeltReminder_SBR_Beifahrer = nicht_aktiv

ASD Module:

Disable Active Sound Design

3000 Funktionen-Baureihe  set to: F021

CMB_MEDIA: (Only for vehicles WITHOUT Navigation)

Transfer Telephone Ringtone to car:

3001  INBAND_RINGING = aktiv 01

FEM_BODY Module:

Start Car by pushing button (hold clutch/brake not needed)

3020TCM_STARTLOCK_BRAKE			set to: nicht_aktiv

3020TCM_STARTLOCK_CLUTCH			set to: nicht_aktiv

3020TCM_STARTLOCK_DRIVINGREADINESS	set to: nicht_aktiv

Increase Angel Eyes Brightness to 100%

3062  MAPPING_STANDL_V_L_PWM_LEVEL_1 = 100%

3062 MAPPING_STANDL_V_R_PWM_LEVEL_1 = 100%

3062  MAPPING_PARKL_V_L_PWM_1_LEFT = 100%

3062 MAPPING_PARKL_V_L_PWM_2_RIGHT = 100%

Close windows, sunroof & mirror from key fob:

3053  KOMFORTSCHLIESSUNG_FB  = aktiv 00

3053  KOMFORT_SCHLIESSEN  WERTE = 00  (Mirror Fold Delay = 0 sec)

3110  ASP_BEIKLAPPEN = aktiv (werte=01)

3110  ASP_BEIKLAPPEN_BEI_KOMFORTSCHLIESSEN = (werte=01)

3110  ASP_AUSKLAPPEN_NACH_KOMFORTSCHLIESSEN = (werte=01)

Change Mirror Un-Fold Automatically at XX Speed (Binary value in Werte):

3110  ASP_GESCHWINDIGKEIT_AUTO_AUSKLAPPEN  werte=14 (12Mph)

Turn off MMI when door is opened:

3020  TCM_LOGIC_R_OFF_DOOR					set to: aktiv

Auto Start/Stop Default OFF:

3023  TCM_MSA_DEFAULT_OFF					set to: aktiv

Auto Start/Stop Remember Last Setting:

3023  TCM_MSA_MEMORY					set to: aktiv

Unlock Door on Engine OFF (1 pull):

3040  CLM_UNLOCK_KL15OFF_AFTER_PIA_AUTO_LOCK	set to: aktiv

Turn off window roll-up door interrupt:

3050  FH_TUERAUF_STOP_MAUT		 	set to: nicht_aktiv

Change Triple-Blink Count to another Value:

3060  BLINKZYKLEN_ANZAHL_TIPP – Werte = # blinks  minus  1

(ie. 4 blinks = werte=3)

Turn on Brake Force Flashing with Hard Braking:

3060  ESS_AKTIVIERBARER_AUSGANG = bremsclicht blinkend

Turn on Fog Lights with Welcome Lights:

3063  MAPPING_NEBELSCHW_L_PART_OF_WL = hard on

3063  MAPPING_NEBELSCHW_R_PART_OF_WL = hard on

Turn off Amber Side Markers next to Headlight:

3063  MAPPING_SIDEMARKER_L_OUTPUT = OFF

3063  MAPPING_SIDEMARKER_R_OUTPUT = OFF

Tilt Passenger Mirror farther down when in reverse

3110ASP_BORDSTEINAUTOMATIK_DELTA	set to: werte=4B

Ambiance Lighting controlled independent from Dimmer Switch

3070  AMBIENTE_NACHFUEHRUNG		set to: nicht_aktiv

Enable Illumination of Exterior Door Handle LEDs when in Reverse

3070OVT_BEI_REUCKFAHRLICHT		set to: aktiv

Disable Headlight Washers

3080  WW_SCHEINWERFERREININUNG	set to: nicht_aktiv

Auto Light Sensitivity Increase

3130RLS_DEF_FLC_SCHWELLWERT_SATZ  	set to: option below
3130RLS_FLC_SCHWELLWERT_SATZ 		set to: option below
Options:
normal = mornal
empfindlich = sensitive
sehr_empfindlich = very sensitive
unempfindlich = insensitive

HU_ENTRY / HU_NBT Module: (Not all options available in HU_ENTRY)

Set cell phone ringer as primary

3003INBAND_RINGING				set to: aktiv – werte=01

Disable Auto-Locking of Doors once begin driving

3000DOOR_LOCK_AFTER_DRIVING_AWAY	set to: nicht_aktiv

Enable ///M LapTmer

3000  M_LAPTIMER				set to: aktiv

Disable Active Sound Design

3000  ASD_SOUND_OFF				set to: aktiv

Enable Active Sound Design Configuration Menu

3000  ASD_CONFIGURATION			set to: aktiv

3000  ASD_SOUND_4				set to: aktiv

Enable Turn Signals in HUD

3001HUD_TURNSIGNAL				set to: aktiv

Change Navi Voice to British English

3000LANGUAGE_ENGLISH_UK			set to: master

3000LANGUAGE_ENGLISH_US			set to: nicht_aktiv

Navi Trip-Import

3000NAVI_TRIP_IMPORT				set to: aktiv

Enable Video Files from USB

3000ENT_CODEC_XVID				set to: aktiv

3000ENT_CODEC_XVCD				set to: aktiv

3000ENT_CODEC_OGG				set to: aktiv

DVD-in-Motion activated:

3000  SPEEDLOCK_X_KMH_MAX  		werte = FF

3000  SPEEDLOCK_X_KMH_MIN 	 		werte = FF

3000  VIDEO_HANDBRAKE			set to: nicht_aktiv

3000  VIDEO_FRONT_LOCKED			set to: nicht_aktiv

3000  VIDEO_SPEEDLOCK_CONDITION		set to: none=00

Remove Legal Disclaimers:

3001  LEGAL_DISCLAIMER_TIME		set to: kein_ld

3001  MACRO_CAM_LEGALDISCLAIMER	set to: kein_ld

Display Tire Pressure & Temperaure in Vehicle Info:

3001  RDC_SAFETY				set to: aktiv 01

Add Weatherband Radio to sources:

3002  RADIO_WEATHERBAND			set to: aktiv 01

Enable the function of saving video files to NBT Hard Drive for play later

3000  ENT_MC_VIDEO_SUPPORT			set to: aktiv

Disable Office Function Speed Lock-Out

3000 OFFICE_MESSAGES_SPEEDLOCK_CONDITION	set to: nicht_aktiv

Enable Full Text Lines in Office While Driving

3003  PIM_DRIVING_TEXT_LENGTH		set to: whole text_07

Disable Audible Chirp/Beep When Locking Car

3000  ACOUSTICAL_LOCK_CONFIRM		set to: nicht_aktiv

Enable Speeach Input for E-Mail or SMS

3003  SPEECH_2_TEXT				set to: aktiv

Enable Time-Stamp Display for Call Log

3003  TIMESTAMP_DISPLAY		set to: aktiv

Add GPS Current Location to Navi Menu Items

3000NAVI_POSITION_MENU		set to: aktiv

Enable IPCE Web Server @ 172.16.222.1

3000IPCE_WEBSERVER			set to: aktiv

Enable WiFi Hot-Spot

(Need to install a DLNA/UPnP media sharing server on your laptop or phone to be able to use this)

3000ENT_MC_WLAN_STREAM_DMP_COVERART	set to: aktiv

3000ENT_MC_WLAN_STREAM_DMP_VIDEO		set to: aktiv

3000ENT_MC_WLAN_STREAM_DMP			set to: aktiv

3003WLAN_STATUS					set to: aktiv

3003WLAN_MODUS					set to: ap

3003WLAN_SIGNAL_STRENGTH				set to: outdoor

3003WLAN_CHANNEL					set to: channel_1 -14

Enable Developer Menu

3000  ENTWICKLER_MENUE		set to: aktiv 01

IHKA Module:

A/C System Remembers Last Setting

3003MEMORY_OFF			set to aktiv-werte=01

Always Remember Air ReCirculating

3003MEMORY_UMLUFT			set to: aktiv

KOMBI Module:

Show Digital Speed in MPH in Cluster

3000BC_DIGITAL_V			set to: aktiv-werte=01

Show Turn Signals in HUD

3008HUD_PIA_BLINKER			set to: aktiv

3000HUD_BLINKER_ENABLE		set to: aktiv

Change Startup Screen in SpeedoLCD to ///M Performance:

3000  BMW_LOGO = mpm

Enable GPS-Sync for Clock

3000GPS_UHR				set to: aktiv

Disable HUD Startup Animation

3008HUD_STARTUP_TIME		Werte=00

REM Module:

Turn On Tail Lights with DRLs:

3062  MAPPING_TAGFAHRL_H_L_OUTPUT = sl_l

3062  MAPPING_TAGFAHRL_H_R_OUTPUT = sl_r

3062  MAPPING_TAGAHRL_H2_L_OUTPUT = sl_2_l

3062  MAPPING_TAGAHRL_H2_R_OUTPUT = sl_2_r